REPORT TO

FIVE RIVERS METROPARKS

Section 10: Island MetroPark

March 21, 2014

Legend of Abbreviations

aff…	   Above the finished ground

AR…	   Accessible Route

CIL…	   Change in Level

CFS…   Clear Floor Space

CGS…   Clear Ground Space

EPC…   Elevated Play Component

GLPC…Ground Level Play Component

Background

The RAC staff visited Island MetroPark to conduct an access audit. The findings are below.

There are three format additions in this report.  We included cells for these items: responsible employee (required by regulation), progress towards completion (best practice), and date of completion (required by regulation).  We have added the projected costs of work as a best practice, and costs are found in the transition grid.

  - Parking - [CHECKLIST] slopes- 2.5% first space IM1, IM2, 3.6% aisle IM3, IM4, 2.7% second space IM5, IM6; signage lacks violation (checklist); signs 43” IM7, IM8

Recommendations (Island MetroPark is a site designated with accessible features so 1.1.1 through 1.1.3 is integral to compliance with title II program access test):

    - Repair or correct slope of parking spaces and access aisles to max 2.08% in any direction (IM1, IM2, IM3, IM4, IM5, IM6)

    - Acquire and mount correct violation signs for all stalls per state or local mandate (checklist)

    - Raise existing accessible parking signs so lowest end of bottom sign is min 60” aff (IM7, IM8)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Accessible Route - [CHECKLIST-EAR] [CHECKLIST-STAIRS]

EAR - curb ramp running slope 8.9% IM39, IM40; curb ramp lacks detectable warning IM40; pea gravel on route behind band shell IM343; deterioration on right side of path IM51, IM57; sand and dirt on route by bridge IM58, IM59, IM60; shifted pavers at kayak area IM348; .5” CIL near splash pad IM44, IM45; .5” CIL toward 5-12 play areas IM46, IM47; .75” gap near front restroom IM41, IM42; deterioration near restrooms and splash pad IM43; cross slopes: 4.6% on route to band shell IM50, IM51; 4.1% to back restrooms IM56, IM57; 2.6% near Chestnut Shelter IM61, IM62; 3.3% under bridge IM63, IM64; and 3.6% along river IM65, IM66

Recommendations (Island MetroPark is a site designated with accessible features so 1.2.1 through 1.2.8 is integral to compliance with title II program access test): (EAR)

    - Correct curb ramp running slope to max 8.33% (IM39, IM40)

    - Install compliant detectable warning at curb ramps as a smart practice (IM40)

    - Train staff to keep surface of AR free of debris (IM343, IM58, IM59, IM60)

    - Resurface AR where route is deteriorating (IM51, IM57, IM43)

    - Repair and reset pavers to eliminate CIL and gaps (IM348)

    - Repair, bevel, or ramp CILs along AR (IM44, IM45, IM46, IM47)

    - Correct or fill gap along AR (IM41, IM42)

    - Correct or repair sidewalk cross slope along AR to max 2.08% (IM50, IM51, IM56, IM57, IM61, IM62, IM63, IM64, IM65, IM66)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Stairs -

Risers: bandshell front left- top step 6.5”, rest 6” IM215, IM216, IM217; band shell rear left- steps vary 5.5”-6.5” IM220, IM221, IM223; band shell rear right- bottom step 4.5”, rest 5.75” IM227, IM228, IM229; and additional step at top IM233, IM234

Treads: bandshell front left- top step 11”, rest 12” IM218, IM219, IM217; band shell rear left- top tread 9.5” deep IM222, IM223; band shell rear right- treads vary 11.5”-12” IM230, IM231, IM232; band shell interior (right/left)- treads 9.5” deep IM205, IM206; all lack detectable warnings at top of runs (checklist)

Handrails: bandshell front (left/right)- lack handrails IM214, IM217; band shell rear left- handrails lack extensions IM226, IM227; handrails mounted 31.5” aff IM224, IM226; and 1” clearance between handrail and wall IM225, IM226; band shell rear right- handrail on one side only IM229; lack extensions IM237; handrail mounted 32.5” aff IM236, IM237; and 1” clearance between handrail and wall IM238, IM239; band shell interior (right/left)- lack extensions IM208, IM212; mounted 33.5” aff IM207, IM208; and lack 80” overhead clearance IM209, IM210

Recommendations (Island MetroPark is a site designated with accessible features so 1.2.1 through 1.2.5 is integral to compliance with title II program access test): (Stairs)

    - Correct riser heights on stairs so they are a consistent height between 4” to 7”; leave as is if technically infeasible (IM215, IM216, IM217, IM220, IM221, IM223, IM227, IM228, IM229, IM233, IM234)

    - Correct tread depth on stairs so they are 11” minimum and consistent, leave as is if technically infeasible (IM218, IM219, IM217, IM222, IM223, IM230, IM231, IM232, IM205, IM206)

    - Install detectable warning strip on top tread of each stairway as a smart practice (checklist)

    - Install handrails on both sides of all stairways, 34” to 38” aff with top and bottom extensions and having a 1.25” – 2” in diameter, or a non-circular grip that has a perimeter dimension of 4”- 6.25” max (IM214, IM217, IM226, IM227, IM224, IM225, IM229, IM237, IM236, IM238, IM239, IM208, IM212, IM207)

    - Provide 80” overhead clearance at band shell stairs if feasible, in the alternative, pad the obstruction to prevent injury (IM209, IM210)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Playground Designated Entry - [CHECKLIST-1] [CHECKLIST-2] [CHECKLIST-3]

5 to12 (Large): no issues

5 to 12 (Small): no issues

2 to 5: no issues

  - Playground Surface/Accessible Route within - [CHECKLIST-1] [CHECKLIST-2] [CHECKLIST-3]

5 to 12 (Large): lacks 80” overhead clearance at GLPC and arches, obstructs route to some EPC egress IM337, IM338

5 to 12 (Small): 3.2% slope to double slide IM324, IM325; lacks 80” overhead at GLPC (checklist)

2 to 5: no issues

Recommendations (Island MetroPark is a site designated with accessible features so 1.4.2 is integral to compliance with title II program access test):

    - Leave as is, lack of 80” overhead clearance at large and small 5 to 12 play areas; correction may be technically infeasible (IM337, IM338, checklist)

    - Repair or correct cross slope of play area accessible surface to max 2.08% (IM324, IM325)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Transfer System - [CHECKLIST-1] [CHECKLIST-2] [CHECKLIST-3]

5 to 12 (Large): 3.4% slope at one transfer IM297, IM298

5 to 12 (Small): 2.9% slope at transfer IM314, IM315

2 to 5: lacks transfer, 4” CIL to low platform IM327, IM328

Recommendations (Island MetroPark is a site designated with accessible features so 1.5.1 is integral to compliance with title II program access test):

    - Repair or correct slope of play area accessible surface to max 2.08% for a level CFS at transfer systems (IM297, IM298, IM314, IM315); in the alternative, leave as is 5 to 12 large play area

    - Install transfer system on 2 to 5 play structure with access to 50% of the EPCs, if feasible (IM327, IM328)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO, METROPARKS OPTION |

  - Ramps - [CHECKLIST-1] [CHECKLIST-2] [CHECKLIST-3]

5 to 12 (Large): >20 EPC, lacks ramp (checklist)

5 to 12 (Small): not applicable

2 to 5: not applicable

Recommendations (in the alternative, leave as is and designate playgrounds at other sites in the system as accessible):

    - Play structure has 20 or more EPCs, install ramp to large 5 to 12 structure to access 50% of the play elements or remove EPCs to reduce total to under 20 (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | METROPARKS OPTION |

  - Elevated Play Components - [CHECKLIST-1] [CHECKLIST-2] [CHECKLIST-3]

5 to12 (Large): 18 of 21 components compliant (checklist); one step 8.25” aff IM302, IM303; apply incentive scoping due to lack of ramp (checklist)

5 to 12 (Small): 8 of 10 components compliant (checklist); apply incentive scoping due to lack of ramp (checklist)

2 to 5: 0 of 8 compliant due to lack of transfer (checklist); 5”-6” CILs between platforms IM328; apply incentive scoping due to lack of ramp (checklist)

Recommendations (in the alternative, leave as is and designate playgrounds at other sites in the system as accessible):

    - Leave as is, 8.25” step on large play area; cite construction tolerance (IM302, IM303)

    - See 1.5.2 above for access to EPC’s on 2 to 5 structure (IM328, checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | METROPARKS OPTION |

  - Ground Level Play Components - [CHECKLIST-1] [CHECKLIST-2] [CHECKLIST-3]

5 to 12 (Large): fails incentive scoping (checklist); lacks accessible GLPC IM310, IM311, IM312, IM313; requires seven GLPC of four different types (checklist)

5 to 12 (Small): fails incentive scoping (checklist); requires one more type of GLPC (checklist); lacks accessible climber IM322, IM323

2 to 5: fails incentive scoping (checklist); requires two more types of GLPC (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.8.3 is integral to compliance with title II program access test):

    - Add seven GLPC’s of 4 different types such as climbers, spinners or rockers, to meet one of each type accessible and incentive scoping for large 5 to 12 structure (checklist); in the alternative, leave as is and designate playgrounds at other sites in the system as accessible (checklist)

    - Leave as is existing GLPC’s at large 5 to 12 structure as correction may be technically infeasible (IM310, IM311, IM312, IM313)

    - Add two more types of GLPCs to the small 5 to 12 structure such as climbers, spinners or rockers, to meet incentive scoping; leave as is existing climber (IM322, IM323)

    - Add two more types of GLPCs to the 2 to 5 structure such as climbers, spinners or rockers, to meet incentive scoping (checklist); in the alternative, leave as is and designate playgrounds at other sites in the system as accessible

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO, METROPARKS OPTION |

  - Sand box/Play tables - [CHECKLIST-1] [CHECKLIST-2] [CHECKLIST-3]

5 to 12 (Large): not applicable

5 to 12 (Small): no issues

2 to 5: no issues

  - Park Site - [CHECKLIST]

Mutt Mitt: located off route IM341

Band Shell: .5” CIL to paved audience area IM53, IM54; gaps at sewer grate IM55; pea gravel on route to rear (see EAR); lacks AR to stage (checklist)

Grills (7): lack routes IM346, IM347, IM352

Kayak Area: 18.3% slope to water IM348, IM349, IM350

Face Cutout: located off route IM351

Splash Pad: 5.4% slope approaching IM353, IM354; chain pulls high and require tight pinch or grasp IM355

Drinking Fountain: single bowl IM339

Portable Toilet: not accessible design IM340

Recommendations (Island MetroPark is a site designated with accessible features so 1.10.1 through 1.10.10 is integral to compliance with title II program access test):

    - Relocate mutt mitt to be along AR (IM341)

    - Repair or bevel CIL to paved area at band shell (IM53, IM54)

    - Replace sewer grate at band shell with one having gaps max .5” (IM55)

    - Create AR to stage level at band shell (checklist)

    - Extend AR to 20% of the grills as a smart practice (IM346, IM347, IM352)

    - Leave as is, slope to water at kayak area and assure that an AR connects from parking to the adjacent loading area (IM348, IM349, IM350)

    - Extend AR to face cut out and provide a 30” by 48” CFS around the sign for use (IM351)

    - Correct or repair slope of AR to splash pad to max 5% (IM353, IM354)

    - Replace at least one chain pull at splash pad with a handle usable without a tight pinch or grasp and within reach range of max 44” agl (IM355)

    - Replace drinking fountain with a hi-lo bowl fountain (IM339)

    - Replace portable toilet with a compliant ADA model (IM340)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Shelters/Picnic Areas - [CHECKLIST]

Stillwater Shelter: 4.1% slope at table clear space IM356, IM357; 1” gap to small grill IM362, IM363; 1” CIL to large grill IM358, IM359; garbage can openings face away from route (checklist); 4.1% slope at large grill IM360, IM361

Chestnut Shelter: tables placed improperly IM364; .75” gap at center and 1” gap at crack IM366, IM367, IM368, IM369; table placement blocks route to amenities (checklist); grill and garbage lack route IM365

Bald Cypress Shelter: 5.7% slope to one section IM370, IM371; brochure box 68” aff IM373, IM374

Willowwood Shelter: 0 of 4 tables accessible IM375; outlet blocked by table IM378; brochure rack mounted high IM376, IM377

Red Bud Shelter: outlet blocked by table IM380; brochure rack mounted high IM379

Recommendations (Island MetroPark is a site designated with accessible features so 1.11.1 through 1.11.9 is integral to compliance with title II program access test):

    - Correct or repair slope in CFS of table and grill in Stillwater shelter to max 2.08% (IM356, IM357, IM360, IM361)

    - Correct or fill gaps to grill at Stillwater and on Chestnut shelter interior (IM362, IM363, IM366, IM367, IM368, IM369)

    - Repair, bevel or ramp CIL on Stillwater shelter interior to grill (IM358, IM359)

    - Reposition garbage can at Stillwater shelter to face the AR (checklist)

    - Relocate tables to provide CFS at amenities throughout shelter interiors and to provide access to the accessible portions of the tables (IM364, IM378, IM380, checklist)

    - Extend AR to grill and garbage at Chestnut shelter (IM365)

    - Correct or repair running slope at Bald Cypress shelter to max 5% (IM370, IM371)

    - Lower brochure rack in Bald Cypress, Willowwood and Red bud shelters to max 48” aff to the highest operable part (IM373, IM374, IM376, IM377, IM379)

    - Replace 20% of the picnic tables with ones with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around table in the Willowwood Shelter (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Park Buildings

Doors - [CHECKLIST]

Maneuvering Clearance: chase- interior equipment, gaps at floor grate IM10; band shell stage right- wall on push side IM67; and stairs on both sides IM67, IM72, IM73; band shell stage left- wall on push side (checklist); and stairs on both sides (checklist); women’s back restroom- 2.7% slope IM78, IM79

Doors Dimensions (32” x 80”): band shell stage doors (R/L): 77.5” to closer IM68, IM69

Change in Level: women’s back restroom- .5” IM76, IM77

Thresholds: band shell stage doors (right/left): lip IM71

Gaps: men’s back restroom- 1” IM74, IM75

Hardware: chase- lacks knurled handle (checklist); women’s front restroom- handle limits space IM9; band shell stage doors (right/left)- knobs IM70

LBF: 4 of 4 doors with closers exceed 8.5 lbf (checklist)

Closer: 4 of 4 doors with closers close too quickly (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.12.1 through 1.12.13 is integral to compliance with title II program access test): (Doors)

    - Leave as is, lack of clearance around chase entry; correction may be technically infeasible (IM10)

    - For doors along the public circulation route, remove the closer on the stage doors to allow for maneuvering clearance on the push side (IM67)

    - Leave as is, stairs in maneuvering clearance; correction is technically infeasible (IM67, IM72, IM73)

    - For all doors along the public circulation route, correct or repair slope in doorway maneuvering clearance to max 2.08% (IM78, IM79)

    - If 1.12.2 above is done, the closers will not be an issue (IM68, IM69)

    - For doors along the public circulation route, repair, bevel, or ramp CIL’s at door entries to max .25” (IM76, IM77)

    - For doors on public circulation route, replace threshold at exterior doors (IM71)

    - For doors on public circulation route, fill and maintain gaps at doorways to max .5” (IM74, IM75)

    - For all doors along the public circulation route, replace hardware with lever hardware where indicated (IM9, IM70)

    - For all doors along the public circulation route, replace hardware on doors to hazardous areas with knurled lever hardware as a smart practice (checklist)

    - For all doors along the public circulation route, inspect, adjust, and maintain 8.5 lbf to open exterior doors as a smart practice (checklist)

    - For all doors along the public circulation route, inspect, adjust, and maintain closing speed on door closers so that doors do not close to 3” faster than 3 seconds when started at 70 degrees (checklist)

    - For employee only doors, make changes upon renovation (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Rooms - [CHECKLIST]

36” Access Aisle: chase/storage IM11; band shell basement- storage obstructs route to stage stairs IM166

60” Turning Space: chase/storage IM11

Floor Surfaces: chase/storage- ladder to lower level IM12, gaps at grate IM11; band shell basement- stairs to stage trap door IM165

Recommendations (Island MetroPark is a site designated with accessible features so 1.12.1 and 1.12.2 is integral to compliance with title II program access test): (Rooms)

    - Employee only areas permit approach, entry, and exit, relocate obstacles such as storage to create AR through chase/storage (IM11, IM166)

    - Employee only areas permit approach, entry, and exit, relocate obstacles to create turning space of 60” in chase/storage (IM11)

    - For all other deficits, leave as is, employee work area pursuant to 2010 Standards 106.5 Defined Terms, until an employee with a disability works here (IM12, IM11, IM165)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Restrooms - [CHECKLIST-MULTI USER] [CHECKLIST-SINGLE USER]

Both (Front): signage door mounted IM25, IM38; lacks accessible family restroom (checklist); mirror- men’s 50” IM26, IM27, women’s 46” IM13, IM14; rear grab bar- men’s not centered correctly IM32, women’s continuous, lacks 24” on open side IM18, IM19; grab bars 37” aff IM31, IM32, IM20, IM21; door 4.25” from adjacent partition IM35, IM15, IM16

Men’s (Front): door lacks compliant hardware IM34; stall 87” deep for inward opening doors IM29; toilet paper too close IM33

Women’s (Front): toilet centerline 19.5” IM17; hand dryer protrudes 7” IM22, IM23

Recommendations (Island MetroPark is a site designated with accessible features so 1.12.1 through 1.12.11 is integral to compliance with title II program access test): (Front)

    - Remount signage on wall, latch side of the door, 48” to baseline of lowest character and 60” to baseline of highest character (IM25, IM38)

    - Create an accessible family or assisted use toilet room (checklist)

    - Lower mirrors so reflective surface is max 48” aff (IM26, IM27, IM13, IM14)

    - Replace rear grab bar in both or extend existing bar to provide 24” on open side of toilet (IM32, IM18, IM19)

    - Remount grab bars in both to 33” to 36” aff to the top of the gripping surface (IM31, IM32, IM20, IM21)

    - Rehang stall doors to be max 4” from adjacent stall partition in both (IM35, IM15, IM16)

    - Replace stall hardware in men’s with hardware operable without a tight pinch or grasp (IM34)

    - Reverse door in men’s stall to swing outward  (IM29)

    - Remount toilet paper dispenser in men’s max 7” to 9” from front of toilet (IM33)

    - Remount toilet in women’s to 16” to 18” from the side wall to centerline (IM17)

    - Relocate or recess hand dryer in women’s to not interfere with general circulation path, protrusions can’t be greater than 4” (IM122, IM23)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Both (Bandshell): sink bowls obstruct knee clearance IM248, IM249, IM253, IM254; faucet controls fail IM249, IM254; exposed pipes lack insulation IM249, IM254; paper towel dispensers mounted high IM250, IM251, IM256; toilet stalls not wide enough IM241, IM252; continuous grab bars fail, pipes obstruct compliant bars IM242, IM243, IM244, IM245, IM252; stall door mountings fail IM241, IM252; toilet paper dispensers mounted on incorrect walls IM241, IM252

Men’s (Bandshell): toilet seat 16.5” aff IM240, IM241; flush controls mounted on incorrect side IM241; stall door not self closing (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.12.1 through 1.12.10 is integral to compliance with title II program access test): (Bandshell)

    - Raise sink bowl to provide 27" knee clearance, in the alternative, acquire a shallower model in both (IM248, IM249, IM253, IM254)

    - Replace faucet controls with lever controls (IM249, IM254)

    - Insulate exposed pipes under one sink in both (IM249, IM254)

    - Lower paper towel dispenser to max 48” to highest operable part in both (IM250, IM251, IM256)

    - Enclose pipes in restrooms and build compliant stalls having min 60” clear width, 59” depth and compliant grab bars in both (IM241, IM252, IM242, IM243, IM244, IM245)

    - Rehang stall door to be max 4" from adjacent stall partition and on the stall wall farthest from the toilet in both (IM241, IM252)

    - Remount toilet paper dispenser on wall adjacent to toilet, max 7” to 9” from front of toilet, 15” to 48” aff and min 12" above or 1.5" below grab bar (IM241, IM252)

    - Replace toilet seat or replace toilet to 17” to 19” aff in M (IM240, IM241)

    - Replace toilet tank with one having flush mechanism on the open side, in the alternative, install an auto flush unit in men’s (IM241)

    - Adjust stall door to be self-closing in men’s (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Both (Back): side wall grab bars extend 49.5” from rear wall IM257, IM258, IM270, IM271; mirrors mounted high, able to tilt but out of reach range IM261, IM262, IM266, IM267, IM272, IM273, IM274; hand dryers protrude IM268, IM269, IM275

Men’s (Back): rear wall grab bar 36.25” aff IM259, IM260; light switch mounted 70.5” aff IM263, IM264

Recommendations (Island MetroPark is a site designated with accessible features so 1.12.1 through 1.12.5 is integral to compliance with title II program access test): (Back)

    - Replace side grab bar with a 42” long grab bar, mounted max 12” from the rear wall on the close end and 54” to the end of the bar and 33” to 36” aff in both (IM257, IM258, IM270, IM271)

    - Lower mirror so reflective surface is max 40” aff or tilt and secure permanently in both (IM261, IM262, IM266, IM267, IM272, IM273, IM274)

    - Relocate or recess hand dryers to not interfere with general circulation path, protrusions can’t be greater than 4” (IM268, IM269, IM275)

    - Leave as is, grab bar height in M; cite construction tolerance (IM259, IM260)

    - Lower light switch in men’s to max 48” aff  (IM263, IM264)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Other - not applicable

CENTRAL MAINTENANCE –

COMMON AREAS

  - Parking - [CHECKLIST] lacks accessible spaces in public lot (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test):

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping using a high quality paint (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Accessible Route - [CHECKLIST-EAR] 3” CIL from parking to office entry IM48, IM49

Recommendations (Island MetroPark is a site designated with accessible features so 1.2.1 is integral to compliance with title II program access test):

    - Repair, bevel, or ramp CIL along AR (IM48, IM49)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Entry Signage - [CHECKLIST] lacks entry signage (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.3.1 and 1.3.2 is integral to compliance with title II program access test):

    - Mount signage at inaccessible entrances directing patrons in wheelchairs to accessible entrance (checklist)

    - Mount signage at entrance designating it as accessible (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Entry Doors - [CHECKLIST]

Maneuvering Clearance: garage exit near overhead to front- storage on push side IM80; and steps back on push side IM80; garage exit to back- storage on push side IM84; and 8.2% slope on push side IM83, IM84; garage exit to chemical storage- storage on both sides IM85; and cracking IM86; garage exit toward street- bollard on pull side IM99; and 4.2% slope IM100, IM101; garage ramped exit- railing on push side IM102; open garage exit- recessed 10” on push side IM105, IM106; and 2.7% slope on pull side IM107, IM108; office front door- shelf IM141; and recession on push side IM142, IM143; and 2.3% exterior slope IM144, IM145

Change in Level: garage exit to chemical storage- board IM86; and 1” at crack IM89, IM90; garage ramped exit- .75” IM103, IM104; open garage exit- 1” IM111, IM112

Threshold: 5 doors have lipped threshold IM81, IM112

Gaps: garage exit toward street- plant growth IM101

Hardware: garage exit to chemical storage- small on pull side IM87

LBF: 5 of 5 doors with closers exceed 8.5 lbf (checklist)

Closer: 4 of 5 doors with closers close too quickly (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.4.1 through 1.4.13 is integral to compliance with title II program access test):

    - For doors along the public circulation route, relocate storage, furniture, and other obstacles to create 60” maneuvering space around doors (IM80, IM84, IM85, IM99, IM141)

    - For doors along the public circulation route, ramp steps at garage exit and provide a 60” by 60” landing at the door (IM80)

    - For doors along the public circulation route, correct or repair slope at doorway landing to max 2.08% in any direction for level CFS (IM83, IM84, IM100, IM101, IM107, IM108, IM144, IM145)

    - For doors along the public circulation route, resurface CFS at doors for smooth maneuvering space (IM86)

    - For doors along the public circulation route,  widen ramp and relocate railing to provide 12” clearance on the latch side of door (IM102)

    - Leave as is, recession at doors; correction may be technically infeasible (IM105, IM106, IM142, IM143)

    - For doors along the public circulation route, repair, bevel, or ramp CIL at door entries to max .25”  (IM86, IM89, IM90, IM103, IM104, IM111, IM112)

    - For doors along the public circulation route, replace threshold at exterior doors (IM81, IM112)

    - For doors along the public circulation route, maintain doorways to be free of plant growth and correct or fill gaps (IM101)

    - For doors along the public circulation route, replace hardware with lever hardware where indicated (IM87)

    - For doors along the public circulation route, inspect, adjust, and maintain 8.5 lbf to open exterior doors as a smart practice (checklist)

    - For doors along the public circulation route, inspect, adjust, and maintain closing speed on door closers so that doors do not close to 3” faster than 3 seconds when started at 70 degrees (checklist)

    - For employee only doors; make corrections upon renovation (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Elevator or Lift - not applicable

CENTRAL MAINTENANCE B

  - Interior Doors - [CHECKLIST]

Maneuvering Clearance: decoration/cone storage- storage on pull side IM93; restroom- storage on push side (checklist)

Dimension (32"X80"): decoration/cone storage- 29.5” IM91, IM92

Hardware: two doors have knobs IM98

Side Lights: decoration/cone storage- 43.25” side IM94, IM95; and 44.5” on door IM96, IM97

Recommendations (Island MetroPark is a site designated with accessible features so 1.6.1 through 1.6.4 is integral to compliance with title II program access test):

    - For doors along the public circulation route, relocate storage and other obstacles for 60” maneuvering space around doors (IM93, checklist)

    - For doors along the public circulation route, replace doors with doors having 32" clear width (IM91, IM92)

    - For doors along the public circulation route, replace hardware with lever hardware where indicated (IM98)

    - For doors along the public circulation route, replace doors with viewing panels with ones max 43" aff to the bottom of the viewing panel (IM94, IM95, IM96, IM97)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Interior Accessible Route - [CHECKLIST-INTERIOR] noncompliant drinking fountain (see Restrooms)

Recommendations (Island MetroPark is a site designated with accessible features so 1.7.1 is integral to compliance with title II program access test):

    - Replace drinking fountain with a compliant hi-lo bowl fountain (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Public Designated Use Spaces (includes classrooms, meeting rooms, special purpose rooms, and other spaces intended for use by the public) - not applicable

  - Employee Offices and Spaces - [CHECKLIST]

Vacant Front Area: not occupied, used as storage for auction items, no plan for further use IM167, IM168, IM169, IM170, IM171, IM172, IM173, IM174

60” Turning Space: decoration/cone storage- storage obstructs IM175

Reach Range: storage garage- knobs 57” aff IM176, IM177; and compression hose 71” aff IM178, IM179

Recommendations (Island MetroPark is a site designated with accessible features so 1.9.1 and 1.9.2 is integral to compliance with title II program access test):

    - Employee only area permit approach, entry, and exit, relocate obstacles such as tables and chairs to create AR through vacant area (IM167, IM168, IM169, IM170, IM171, IM172, IM173, IM174)

    - Employee only areas permit approach, entry, and exit, relocate obstacles to create turning space of 60" in storage (IM175)

    - For all other deficits, leave as is, employee work area pursuant to 2010 Standards 106.5 Defined Terms, until an employee with a disability works here (IM176, IM177, IM178, IM179)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Assembly Areas - not applicable

  - Restrooms -  [CHECKLIST-MULTIPLE USERS]

Vacant Restrooms: not audited, not accessible and not used (checklist)

Maintenance: lacks compliant signage (checklist); lacks accessible toilet stall IM283, IM284; noncompliant drinking fountain IM285

Recommendations (Island MetroPark is a site designated with accessible features so 1.11.1 through 1.11.4 is integral to compliance with title II program access test): (Multiple Users)

    - If renovated for future use, assure vacant restrooms are brought into compliant with the 2010 standards (checklist)

    - Upon renovation, acquire and mount signage, including access symbol, mounted on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character  (checklist)

    - Create a wheelchair accessible stall with grab bars and fixtures mounted in correct locations and at correct heights (IM283, IM284)

    - See 1.7.1 above for corrections to the fountain (IM285)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Kitchen/Concessions - not applicable

  - Locker Rooms - not applicable

  - Aural and Visual Alarms - [CHECKLIST] lacks alarm system

Recommendations (Island MetroPark is a site designated with accessible features so 1.14.1 is integral to compliance with title II program access test):

    - Upon renovation install audible and visual alarms in all rooms and spaces (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Directional and Permanent Space Signs - [CHECKLIST] lacks compliant signage (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.15.1 through 1.15.3 is integral to compliance with title II program access test):

    - Create template for signs that addresses height of sign, size of characters, location of Braille, and other requirements (checklist)

    - Implement a sign revision program throughout the building, discriminating between directional signs and signs for permanent spaces (checklist)

    - Mount signage at all permanent rooms/spaces having Braille and the international symbol of accessibility, mounted at 60" to the middle of the sign and on the latch side of the door (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Other - not applicable

CENTRAL MAINTENANCE A

  - Interior Doors - [CHECKLIST]

Maneuvering Clearance: RR- wall on pull side IM114; garage to break room- garbage can on push side (checklist); break room to hall- table back on pull side IM120; garage storage W locker room- locker on pull side IM121; W locker room to front office- table on pull side IM122; garage to front office- wall on pull side IM125; front door- storage on pull side IM140; and hall too narrow IM135, IM136; first hall office- storage interior IM132; hall too narrow IM133, IM134; hall closet- no entry IM150; second hall office- chair on pull side IM152, IM153; and hall too narrow (checklist); third hall office- storage on pull side IM154; W RR - cabinet on pull side IM158; hall narrow (checklist); M RR - water heater on pull side IM157; hall narrow (checklist); office hall to garage by RRs- wall on push side IM160

Dimension (32"X80"): restroom- 29.5” wide IM115, IM116; break room to hall- 29” wide IM119, IM120; first hall office- 29” wide IM130, IM131; hall closet- 77.75” overhead IM148; and 20” wide IM149; second hall office- 29.5” wide (checklist); ladies restrooom- 29.5” wide (checklist)

Change in Level: garage to front office- .75” IM127, IM128; office hall to garage by restrooms- .5” IM159, IM160

Gaps: garage to front office- broken tiles IM128; men’s restroom- 1” IM155, IM156

Hardware: 13 doors fail IM129, IM140, IM132, IM153, IM154; restroom- slide lock high IM117, IM118; hall closet- sliding hardware fails IM151

LBF: 3 of 7 doors with closers exceed 5 lbf (checklist)

Closer: 4 of 7 doors with closers close too quickly (checklist)

Push Side Surface: five doors fail IM137, IM138, IM139

Side Lights: garage to front office- 62” aff IM123, IM124; office hall to garage by restrooms- 48” aff IM161, IM162

Recommendations (Island MetroPark is a site designated with accessible features so 1.6.1 through 1.6.15 is integral to compliance with title II program access test):

    - For doors along the public circulation route, remove and rehang to swing outward not inward for 18” maneuvering clearance on pull side of door (IM114)

    - For doors along the public circulation route, relocate storage, furniture, and other obstacles to create 60” maneuvering space around doors (IM120, IM121, IM122, IM140, IM132, IM152, IM153, IM154, IM158, IM157, checklist)

    - For doors along the public circulation route, remove and rehang door to open from the opposite side for 18” maneuvering clearance on the pull side (IM125)

    - Leave as is, narrow hallways until renovation occur; correction may be technically infeasible (IM135, IM136, IM133, IM134, checklist)

    - Leave as is, hall closet; no entry (IM150)

    - For doors along the public circulation route, remove closer to provide required maneuvering clearance on push side of door (IM160)

    - For doors along the public circulation route, replace doors with doors having 80” overhead clearance and 32" clear width (IM115, M116, IM119, IM120, IM130, IM131, IM148, IM149, checklist)

    - For doors along the public circulation route, repair, bevel, or ramp CIL’s at  door entries to max .25” (IM127, IM128, IM159, IM160)

    - For doors along the public circulation route, fill and maintain gaps at doorways to max .5” (IM128, IM155, IM156)

    - For doors along public circulation route, replace hardware with lever hardware where indicated (IM129, IM140, IM132, IM153, IM154, IM117, IM118, IM151)

    - For doors along the public circulation route, inspect, adjust, and maintain 5 lbf to open interior doors (checklist)

    - For doors along the public circulation route, inspect, adjust, and maintain closing speed on door closers so that doors do not close to 3” faster than 3 seconds when started at 70 degrees (checklist)

    - For doors along the public circulation route, install kick plate covering the bottom 10" of door (IM137, IM138, IM139)

    - For doors along public circulation route, replace with ones with viewing panels max 43" aff to the bottom of the viewing panel (IM123, IM124, IM161, IM162)

    - For employee only doors; make corrections upon renovation (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Interior Accessible Route - [CHECKLIST-INTERIOR] interior route along hallway lacks turning space (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.7.1 is integral to compliance with title II program access test):

    - Leave as is, narrow hallways, until renovations occur; turning space provided within rooms along hallway (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Public Designated Use Spaces - [CHECKLIST] no issues

  - Employee Offices and Spaces - [CHECKLIST]

36” Access Aisle: sliding blue storage- storage obstructs IM182; storage/women’s locker room- 25” at bench IM190, IM191; and 30” at partition IM194, IM195

80” Clearance: print room- 79” to hallway IM200, IM201

60” Turning Space: sliding blue storage- storage obstructs IM182; materials storage- storage obstructs IM184; storage/women’s locker room- bench obstructs IM191; first office- furniture obstructs IM202; second office- furniture obstructs IM204

Protruding Objects: open garage- window frames IM181; sliding blue storage- storage racks IM183

Floor Surfaces: open garage- large grate IM180

Clear Floor Space: print room- time clock IM197

Reach Range: open garage- first aid kit IM188, IM189; and microwave IM187; storage/women’s locker room- hooks IM193; print room- time clock IM196, IM197; and thermostat IM198, IM199

Sink: open garage- 36” aff IM185, IM186

Recommendations (Island MetroPark is a site designated with accessible features so 1.9.1 and 1.9.2 is integral to compliance with title II program access test):

    - Employee only area permit approach, entry, and exit, relocate obstacles such as storage for AR through storage areas (IM182, IM190, IM191, IM194, IM195)

    - Employee only areas permit approach, entry, and exit, relocate obstacles for 60” turning space in storage and offices (IM182, IM184, IM191, IM202, IM204)

    - For all other deficits, leave as is, employee work area pursuant to 2010 Standards 106.5 Defined Terms, until an employee with a disability works here (IM200, IM201, IM181, IM183, IM180, IM197, IM188, IM189, IM187, IM193, IM196, IM198, IM199, IM185, IM186)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Assembly Areas - not applicable

  - Restrooms -  [CHECKLIST-MULTIPLE USERS]

Maintenance: locker storage elements out of reach range IM288, IM289; lacks accessible toilet stall IM287, IM288; noncompliant shower IM288; and noncompliant bench IM289

Both (Hall): lacks compliant signage IM290, IM294; lacks accessible toilet stall, large enough to be converted into accessible single user restrooms IM291, IM292, IM295, IM296

Recommendations (Island MetroPark is a site designated with accessible features so 1.11.1 through 1.11.6 is integral to compliance with title II program access test): (Multiple Users)

    - Designate 5% of the lockers as accessible and lower hooks and shelving to be in reach range 15” to 48” aff (IM288, IM289)

    - Create a wheelchair accessible stall with grab bars and fixtures mounted in correct locations and at correct heights in maintenance (IM287, IM288)

    - Create a compliant accessible shower in the maintenance restroom (IM288)

    - Replace bench in maintenance RR with one with seat depth of  20" to 24” deep, 42” long, affixed to the wall or having a back and mounted 17” to 19” aff (IM289)

    - Create two single user unisex restrooms in place of existing hall bathrooms and renovate one to be of accessible design (IM291, IM292, IM295, IM296)

    - Upon renovation, acquire and mount signage including Braille and access symbol for restrooms (IM290, IM294)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Kitchen/Concessions - not applicable

  - Locker Rooms - not applicable

  - Aural and Visual Alarms - [CHECKLIST] lacks alarm system (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.14.1 is integral to compliance with title II program access test):

    - Upon renovation install audible and visual alarms in all rooms and spaces (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Directional and Permanent Space Signs - [CHECKLIST] lacks compliant signage (checklist)

Recommendations (Island MetroPark is a site designated with accessible features so 1.15.1 through 1.15.3 is integral to compliance with title II program access test):

    - Create template for signs that addresses height of sign, size of characters, location of Braille, and other requirements (checklist)

    - Implement a sign revision program throughout the building, discriminating between directional signs and signs for permanent spaces (checklist)

    - Mount signage at all permanent rooms/spaces having Braille and the international symbol of accessibility, mounted at 60" to the middle of the sign and on the latch side of the door (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Other - not applicable

FIVERIVERSMP ISLNDMTROPRK SEC10